| Issue date: October 2026 | Next review date: October 2027 | Approved by: Andrew Anderson |
| --- | --- | --- |
| On a Total Fire Ban day: NO welding, grinding or cutting outdoors, NO burning off, NO mowing, trimming or slashing dry grass. Other machines near dry grass only for an essential job the Course Superintendent approves, set up to CFA rules. |  |  |
| Applies to | Everyone who works at Ranfurlie and Amstel |  |
| What’s covered | Fire Danger Ratings, the Fire Danger Period, Total Fire Ban rules, hot work, mowers, machinery and vehicles in dry grass, barbecues and smoking, Catastrophic days, bushfire smoke, and what to do if a fire starts |  |
| Time needed | About 12 minutes to read, then a 10-question knowledge check |  |
| Re-sign | Every 6 months, and whenever this document is updated |  |

The 8 things that matter most - Ranfurlie is in the City of Casey, in the Central fire weather district. Check the Fire Danger Rating and whether a Total Fire Ban is declared – VicEmergency app or the CFA website. - Fire Danger Ratings: Moderate, High, Extreme, Catastrophic – plus “no rating” days when no action is needed. - Total Fire Bans are the law. They are announced the afternoon before and run from midnight to midnight. - No hot work outdoors on a Total Fire Ban day – no welding, grinding, cutting or soldering. - No burning off, no fire pits and no wood or charcoal barbecues on a Total Fire Ban day. Gas barbecues only under strict conditions. - No mowing, trimming or slashing dry grass on Total Fire Ban days. Other machines near dry grass only for an essential job the Course Superintendent approves, set up to CFA rules. Green, irrigated turf isn’t restricted by law, but CFA says avoid using machinery wherever possible on TFB days – keep it to essential mowing, with clean machines. - Hot exhausts start fires. Never park or idle a machine or vehicle on long dry grass, and clean grass build-up off machines. - If a fire starts: raise the alarm, call 000 and get away. A grassfire can move faster than you can run.

# 1. Purpose

A golf course in summer has kilometres of dry grass on its edges, machines with hot exhausts, a workshop full of welders and grinders, fuel, and people working in remote areas. This procedure explains the fire danger system, what the law allows on Fire Danger Period and Total Fire Ban days, and how we prevent and respond to fires.

# 2. Fire Danger Ratings

Fire Danger Ratings show how dangerous a fire would be if one starts. They are forecast up to four days ahead for each fire weather district. Ranfurlie (City of Casey) is in the Central fire weather district. Check Central on the VicEmergency app or the CFA website.

| Rating | What it means (CFA) | What it means for us |
| --- | --- | --- |
| No rating | No proactive action needed. Fires can still happen but are unlikely to threaten the community. | Normal work, normal care. |
| Moderate | Most fires can be controlled. Plan and prepare. | Normal work. Keep machines clean and fire equipment ready. |
| High | Fires can be dangerous. Be ready to act. | Extra care with machines in dry grass and with any hot work outdoors. |
| Extreme | Fires will spread quickly and be extremely dangerous. Take action now to protect your life and property. | Your supervisor reviews the day’s jobs. Keep machines out of dry grass and no hot work outdoors. |
| Catastrophic | If a fire starts and takes hold, lives are likely to be lost. For your survival, leave bushfire risk areas. | See section 7. |

# 3. The Fire Danger Period

- The Fire Danger Period (FDP) is declared by CFA for each council area. It usually runs for several months over the hottest part of the year – it can start as early as October and run until autumn. Dates change every year: check “Fire restriction dates” on the CFA website for the City of Casey.

- During the FDP it is illegal to burn off without a permit. Permits are issued by CFA, Fire Rescue Victoria or a Municipal Fire Prevention Officer, depending on the location – apply through Fire Permits Victoria. Council local laws can also apply.

- Staff never burn off. If burning off is ever needed, management arranges the permit and the conditions.

During the FDP, these activities are allowed only if the conditions are met – in the “open air”, which means anywhere not inside a permanent building with a roof and fully enclosed walls:

| Activity | Conditions during the Fire Danger Period |
| --- | --- |
| Welding, grinding, soldering, abrasive cutting discs, gas cutting | - A fire-resistant shield or guard to stop sparks, hot metal and slag - At least 1.5 metres around the work cleared of anything that can burn, or wetted down - A hose connected to mains water, or a water knapsack holding at least 9 litres - Cut-offs and hot material put in fire-proof containers - Someone in attendance the whole time, able to put out a fire – and everything fully out before they leave |
| Mowers, line trimmers, chainsaws in dry (not green) grass | - Free from faults that could start a fire - Fitted with an efficient spark arrester - At least 3 metres around the machine cleared of anything that can burn - The operator stays with it and carries a 9-litre water knapsack or a 9-litre stored-pressure water extinguisher |
| Tractors, slashers, loaders, excavators within 9 metres of grass or other vegetation | - Free from faults that could start a fire - Fitted with a working spark arrester, a turbocharger or an exhaust-aspirated air cleaner - Carrying a 9-litre water knapsack or a 9-litre stored-pressure water extinguisher |
| Vehicles driven through grass | Only with an efficient muffler that takes all the exhaust. Avoid driving through dry grass anyway. |
| Barbecues | Wind under 10 km/h, 3 metres around and above clear of anything that can burn, someone with it at all times, and turned off or fully out when finished. |

A small dry-powder extinguisher does not meet these conditions. The law asks for at least 9 litres of water – a water knapsack or a stored-pressure water extinguisher. Your supervisor will tell you which machines are set up for work in dry grass.

# 4. Total Fire Ban days

- A Total Fire Ban (TFB) is declared by CFA’s Chief Officer for a district, or the whole state, when fires are likely to spread quickly and be hard to control. It can be declared even when the rating is not Extreme.

- It is usually announced the afternoon before, and runs from midnight to midnight.

- FDP permits are suspended on TFB days.

- Breaking a TFB is an offence. CFA says penalties can include fines of up to $46,000 or 2 years’ jail, or both.

- You will see TFB declarations in the VicEmergency app (statewide notices come through even without a watch zone) and on the CFA website.

| Activity | On a Total Fire Ban day |
| --- | --- |
| Burning off | Banned. Any fire already burning must be put out before the ban starts. |
| Welding, grinding, soldering, abrasive cutting discs, gas cutting in the open air | Law: banned. (A special permit is possible only in limited circumstances.) Under CFA rules, an open roller door can make a shed “open air”. Our rule: no hot work outdoors or in open-sided sheds – the job waits. Essential hot work only inside the fully enclosed workshop with all doors closed, and only with the Course Superintendent’s OK. |
| Mowers, line trimmers, chainsaws | Law: in dry grass, only if the work is essential and all the FDP conditions above are met. Green, irrigated turf isn’t restricted by law, but CFA says avoid using machinery wherever possible on TFB days – keep it to essential mowing, with clean machines. Our rule: NO mowing, trimming or slashing of dry grass – including roughs, native areas and boundaries. |
| Tractors, slashers, loaders, excavators within 9 metres of vegetation | Law: avoid wherever possible – only if the work is essential and the machine meets the conditions above. Our rule: near dry grass only for an essential job the Course Superintendent approves, set up to CFA rules. |
| Driving through grass | Law: only with an efficient muffler that takes all the exhaust. CFA advice: avoid driving through dry grass wherever possible. Our rule: stick to paths, tracks and irrigated turf. |
| Barbecues | Gas or electric only, built or commercially made for cooking and stable; 3 metres around clear of anything that can burn; 10 litres of water or a connected hose ready; an adult with it the whole time; fully off before they leave. No wood, charcoal, heat beads or methylated spirits. |
| Fire pits, braziers, incinerators, campfires | Banned. |

Smoking

The TFB rules don’t ban smoking itself, but a dropped butt can start a grassfire. Only smoke where your supervisor says it is OK, well away from dry grass, fuel and chemicals. Put butts out completely in a proper butt bin. Never drop a butt on the ground or flick it out of a vehicle. On TFB days, don’t smoke out on the course.

# 5. Stopping machines from starting fires

Exhausts, catalytic converters and diesel particulate filters run hot enough to set dry grass alight. Grass and dust packed around engines, belts and bearings can smoulder and catch fire. Every day in summer:

- Pre-start: check the exhaust, the spark arrester (if fitted), fuel and oil leaks and wiring. Tag out anything faulty (see OHS-14 Pre-start Checks & Out-of-Service Tagging).

- While working: stop and check for grass build-up around the engine, exhaust, belly and bearings on long jobs.

- Never park or idle a machine, ute or cart on long dry grass. Park on paths, bitumen, concrete or irrigated turf.

- Keep out of dry grass when the fire danger is high – slash and trim dry areas only when the grass is green or the fire risk is lower.

- At the end of the job, blow down the machine, check behind you for smoke or smouldering, and let hot machines cool on a hard surface away from fuel and anything that burns.

- Refuelling: engine off and cooled, no smoking, clean up spills (see OHS-10 Fuel Handling).

- Fire equipment: carry the extinguisher or knapsack fitted to or required for the machine, and know how to use it (see OHS-07 Fire Extinguishers & Fire Wardens).

# 6. If a fire starts

- Raise the alarm. Radio the crew: “Fire on the [hole/area] – everyone stay clear.”

- Call 000. Give the address and the nearest access gate (see OHS-03 Medical Emergency & Ambulance Access).

- Only tackle a small fire if it is safe, the alarm has been raised, you have a clear way out behind you, and you know how to use the extinguisher (see OHS-07). Never try to fight a fire that is spreading, in long grass on a windy day, or if the smoke is thick.

- Get away – early. Grassfires can travel at up to 25 km/h – faster than you can run – and give off intense radiant heat and smoke. Move well away from the fire and its smoke, following OHS-02 Emergency Procedures & Evacuation.

- Don’t drive into smoke. You can crash or get trapped. Keep tracks and gates clear for fire trucks.

- Report in to your supervisor so everyone is accounted for.

A fire nearby, off the course

- Tell your supervisor straight away, and check VicEmergency for warnings.

- Follow any instruction from emergency services and the Course Superintendent. Be ready to stop work and leave.

Bushfire smoke

- Check air quality on EPA AirWatch. WorkSafe Victoria advises cutting back heavy outdoor work when air quality is poor, and avoiding outdoor work when it is very poor or hazardous.

- Paper dust masks and bandannas don’t filter fine smoke. A P2 mask only works if it seals – which it can’t over a beard.

- Wheezing, chest tightness or trouble breathing needs urgent medical attention. People with asthma or heart or lung conditions are more at risk – tell your supervisor.

# 7. Catastrophic days

- CFA’s advice for Catastrophic days: if a fire starts and takes hold, lives are likely to be lost. Homes can’t withstand fires in these conditions, you may not be able to leave, and help may not be available.

- The Course Superintendent will decide the afternoon before whether work goes ahead, and post the plan in #course-team. Expect essential jobs only – on irrigated turf or indoors – no machines in dry grass, no hot work, a minimum crew and an early finish, or no outdoor work at all.

- If you live in a bushfire risk area, follow your own bushfire plan. Tell your supervisor if you need to leave early or can’t come in – you will be supported.

# 8. Scenarios – what should happen?

Scenario 1 – “Just a quick weld”

It is a Total Fire Ban day. A bracket on a mower has cracked and the mechanic wants to weld it in the yard outside the shed so it can go out again after lunch.

What should happen: no. Hot work in the open air is banned on a TFB day. The mower stays tagged out until the next day, or the Course Superintendent decides whether it can be done inside the fully enclosed workshop with all doors closed.

Scenario 2 – “Slashing the boundary”

The job board has slashing the long, dry native grass along the boundary. Overnight, a TFB has been declared for the Central district.

What should happen: the job comes off. Check with your supervisor and swap to work on irrigated turf. It goes back on when the ban is over and the conditions are lower risk.

Scenario 3 – “Smoke behind the Workman”

You parked a Workman on long dry grass beside the 6th to clear a drain. Twenty minutes later you see smoke rising from under it.

What should happen: radio the crew and call 000. If it is tiny, the alarm is raised, you have a clear way out and you know how to use the extinguisher, use it. If it is spreading, get well away from the fire and the smoke. Next time, park on a track or irrigated turf.

Scenario 4 – “Smoko barbecue”

It is a TFB day and someone brings a charcoal barbecue for the crew’s Friday lunch.

What should happen: no charcoal, wood or heat beads on a TFB day. A gas barbecue can be used only if it is stable, has 3 metres cleared around it, 10 litres of water or a hose ready, and an adult with it until it is fully off.

# 9. The law in brief

| Law | What it means for us |
| --- | --- |
| Country Fire Authority Act 1958 (Vic) and regulations | CFA declares Fire Danger Periods and Total Fire Bans, which legally restrict fires and heat-producing activities in the open air. Permits are needed for burning off during the FDP. CFA says penalties can include fines of up to $23,000 or 12 months’ jail during the FDP, and up to $46,000 or 2 years’ jail on a TFB day. |
| Occupational Health and Safety Act 2004 (Vic) – employer duties | The Club must, so far as is reasonably practicable, provide a safe working environment and safe systems of work, and give the information, instruction, training and supervision needed. WorkSafe Victoria has warned that in Total Fire Ban areas, outdoor work that may provide an ignition source, such as welding and other hot works, must not be performed. |
| Occupational Health and Safety Act 2004 (Vic) – your duties | You must take reasonable care for your own safety and the safety of others, and cooperate with this procedure. |

# Document control

- Document: OHS-06 Fire Danger Days & Total Fire Bans – Standard Operating Procedure

- Version: 1 – October 2026

- Owner: Course Superintendent

- Review: every 12 months (before each Fire Danger Period), after any fire or near miss, or when the law or CFA rules change

- Related documents: OHS-02 Emergency Procedures & Evacuation · OHS-03 Medical Emergency & Ambulance Access · OHS-05 Heat Stress & Hot Weather Work · OHS-07 Fire Extinguishers & Fire Wardens · OHS-10 Fuel Handling · OHS-13 Hazard, Incident & Near-Miss Reporting · OHS-14 Pre-start Checks & Out-of-Service Tagging · machinery safe work procedures

- Based on: CFA “About Fire Danger Ratings”, “Can I or can’t I?”, “About Fire Danger Period and Total Fire Ban days”, “About Total Fire Bans”, “Using machinery”, “Fire Danger Period permits”, “Urban grassfires” and “Find your fire weather district”; WorkSafe Victoria fire danger, heat and bushfire smoke guidance; VicEmergency; Country Fire Authority Act 1958 (Vic); Occupational Health and Safety Act 2004 (Vic).