| Issue date: October 2026 | Next review date: October 2027 | Approved by: Andrew Anderson |
| --- | --- | --- |
| DO NOT drive any vehicle or machine you have not been trained and authorised to use – and NEVER carry a passenger in a tray, on a trailer, on a machine or anywhere without a proper seat. |  |  |
| Applies to | Everyone at Ranfurlie and Amstel who drives or rides in a vehicle, cart or machine, or walks around the shed, workshop and car park |  |
| What’s covered | Utility vehicles (Workmans, Gators), carts, mowers moving between holes, tractors, trailers and cars on site – training, speed, paths, golfers, passengers, seatbelts, slopes, loads, reversing, parking, phones, lights, public roads, pedestrians and incidents |  |
| Time needed | About 10 minutes to read, then a 10-question knowledge check |  |
| Re-sign | Every 12 months, and whenever this document is updated |  |

The rules that matter most - Only drive what you’ve been trained and authorised to drive. Do the pre-start check every time (OHS-14). - Drive to the conditions. Follow posted limits. Walking pace near the shed, near people, around the clubhouse and in the car park. - Passengers only in a proper seat – never in a tray, on a trailer, on a mower or standing. - Seatbelt on wherever a rollover protective structure (ROPS) is fitted. - Look before you reverse. If you can’t see, use a spotter – or don’t reverse. - Park safely: level ground, brake on, engine off, key out. - No phone while driving. Pull over and stop first. - Never drive on a public road unless the Course Superintendent has confirmed the vehicle is registered or permitted and you’re licensed. - Zero alcohol. 0.00 BAC whenever you drive or operate machinery (OHS-31).

# 1. What this covers

- Utility vehicles – Workmans, Gators and similar

- Golf carts – staff carts, and members’ and hire carts you share paths with

- Ride-on mowers and other self-propelled machines when travelling between holes and to and from the shed

- Tractors, with or without attachments and trailers

- Trailers towed by any of the above

- Cars, utes and delivery trucks in the car park and around the shed

Each machine also has its own SWP (mowers, utility vehicles, track loader, excavator, sprayer). Follow this procedure and the machine’s SWP.

# 2. Who can drive

- Only drive a vehicle or machine you’ve been trained on and authorised to use by your supervisor. Never had a go on one before? Ask for training first.

- Read and sign the machine’s SWP before you use it.

- Pre-start check every time (OHS-14) – brakes, steering, tyres, seatbelt, ROPS, horn, lights and safety switches. If something is faulty, tag it out and tell your supervisor. Don’t drive it.

- Never bypass a safety feature – seat switches, interlocks, seatbelt alarms or speed limiters. Never clip a seatbelt behind you to stop an alarm: on some side-by-sides this also overrides a speed limiter.

- Be fit to drive – zero alcohol, no impairing drugs or medication, and not overtired (OHS-31).

# 3. Speed – drive to the conditions

- Follow any posted speed limits.

- Drive at a speed that lets you stop safely in the distance you can see. Go slower on wet grass, dew, frost, slopes, gravel, bends, near trees and blind crests, in poor light, and when you’re loaded or towing.

- Walking pace near the shed and workshop, near people (staff, golfers, members, visitors), around the clubhouse and practice areas, and in the car park.

- Speed contributes to side-by-side incidents, especially on uneven ground.

- No racing, skidding, “doughnuts” or mucking around in any vehicle or machine.

# 4. Paths, crossings and golfers

- Use cart paths and service tracks where they exist, unless your job takes you off them.

- Slow right down at path junctions, crossings, blind corners and around buildings. Look both ways and give way to pedestrians.

- Golfers: follow OHS-16 Working Around Golfers. Be seen, make eye contact, wait for shots to be played, and never drive across the line of play.

- Members and hire carts may not be watching for you – expect the unexpected and give them room.

# 5. Passengers

- Passengers ride only in a seat designed for a passenger – one person per seat, and no more passengers than seats.

- Never ride in a tray, on a trailer, on a tow bar, on a mower or tractor, on a footplate or step, or standing up or hanging off the side.

- Passengers wear the seatbelt where one is fitted, and keep arms and legs inside the vehicle. Doors and door nets help keep you inside the rollover zone.

- Follow the manufacturer’s instructions about passengers for that vehicle.

- If there aren’t enough seats, make two trips or walk.

# 6. Seatbelts and ROPS

- Wear the seatbelt whenever the vehicle or machine has a ROPS and it is up and locked. A seatbelt keeps you inside the protected zone if the machine rolls. See OHS-17 and the machine’s SWP.

- Don’t use a vehicle or machine with a damaged ROPS or a faulty seatbelt – tag it out.

- Only fold a ROPS down where the machine’s SWP allows it, and put it back up straight after.

# 7. Slopes, wet ground, banks and water

- Follow the slope rules for your machine in OHS-17 Slopes & Rollover.

- Wet grass, dew, frost and mud cut grip and braking. Slow down, avoid sharp turns, and keep off slopes you’re not sure about.

- Keep well back from bunker faces, banks, drains, lake and dam edges – edges can collapse. See OHS-18 Working Near Water.

- Never let a vehicle or machine coast downhill in neutral.

- If in doubt, park on safe ground and walk the last bit.

# 8. Loads, trays and trailers

- Stay within the load and towing limits on the vehicle’s plate or in the operator’s manual.

- Secure every load – tie it down and keep the tailgate up. Nothing loose that can slide, roll or fly out (tools, bags, flags, cores, sand).

- Keep heavy loads low and centred. A load changes how a vehicle brakes, steers and handles slopes – drive slower, especially downhill and across slopes.

- Trailers: check the hitch, pin and clip before you move. Keep the load balanced. Reverse slowly, with a spotter if you can’t see.

- Tipping trays: check nobody is behind or beside you before you tip. Lower the tray before you drive off.

- Fuel and chemicals: only in approved, closed containers, secured upright (OHS-10, CHM-01).

# 9. Reversing

- Avoid reversing where you can – plan to drive forward out of the spot.

- Check behind before you get in – walk around if needed.

- Look over your shoulder and use your mirrors (and camera if fitted). Remember the blind spots behind trays, trailers and the far side of the vehicle.

- If you can’t see clearly, use a spotter. Agree hand signals first. The spotter stands where you can see them and out of the vehicle’s path. If you lose sight of the spotter – stop.

- Reverse slowly. A reversing beeper doesn’t replace looking.

# 10. Parking and leaving a vehicle

- Park on level ground where you can.

- Park brake on, engine off, key out. Taking the key stops anyone else using it – including members of the public.

- Lower attachments, cutting units and trays.

- If you have to stop on a slope, follow the machine’s SWP (for example, turning the wheels or chocking).

- Don’t block paths, gates, emergency access or the shed doors.

# 11. Phones, radios and distractions

- No calls, texts, Slack or apps while driving – hand-held or not. Pull over safely and stop first.

- Stop to make or answer a radio call when you can, and keep it short.

- Keep your eyes on where you’re going – not on the job board, a map or a sprinkler head behind you.

# 12. Lights and visibility

- Lights on at dawn and dusk, and in fog, rain or smoke. Don’t drive in the dark without working lights.

- On early starts, remember other people can’t see you well – and you can’t see them.

- Low sun at dawn and dusk causes glare. Slow down until you can see clearly.

# 13. Public roads

Never drive any vehicle or machine on a public road – including to cross one, or along a road shoulder or footpath – unless the Course Superintendent has confirmed the vehicle is registered or permitted for that use and you hold the right licence. If a machine needs to go somewhere by road, your supervisor will arrange how it is moved.

Why the rule is strict:

- Golf carts and ride-on mowers are built for off-road use. Victoria’s transport department says they shouldn’t be used to travel on roads, and must not be driven on footpaths, bike paths or shared paths.

- They are only exempt from registration and licensing in narrow conditions – used just for golf or mowing, and not travelling on a road for more than 2 km at a time in one direction. Outside those conditions, normal registration and licence laws apply.

- Golf carts and ride-on mowers aren’t covered by TAC or third-party injury insurance if you have an accident.

- Utility vehicles can only be used on a road if they are registered or have a permit (some can get conditional registration) and every condition is met – including licence, speed, distance and purpose.

# 14. Pedestrians – shed, workshop and car park

If you’re on foot

- Stay out of the path of moving vehicles and machines, and never walk behind a reversing one.

- Make eye contact with the driver before you walk in front of, behind or past a vehicle.

- Stay out of blind spots – beside trays, behind trailers and close to large machines.

- Keep clear of delivery trucks while they are loading and unloading.

If you’re driving

- Walking pace around the shed, workshop and car park. Give way to people on foot.

- Watch for people stepping out between parked cars, staff coming out of the shed, and children in the car park and around the clubhouse.

- Drive your own car in the car park as carefully as you would on a public road.

# 15. If something goes wrong

- Stop and make safe – engine off, key out, brake on.

- If anyone is hurt, give first aid and call 000 for anything serious (OHS-03).

- Tell your supervisor straight away by radio or phone.

- After a serious incident, don’t move vehicles or disturb the scene unless you need to help someone or make the area safe. Management must notify WorkSafe of serious incidents, and WorkSafe may need to inspect.

- Report it in #ohs (a photo helps) and complete the incident form – including near misses, like nearly hitting a golfer or a vehicle sliding on a bank.

- Tag out any damaged vehicle or machine (OHS-14).

# 16. Scenarios – what should happen?

Scenario 1 – “Jump in the back”

Three of the crew need to get to the 14th. The Workman has two seats, so Tom tells the new apprentice to hop in the tray.

What should happen: no-one rides in the tray. Make two trips, take a second vehicle, or walk.

Scenario 2 – “Backing in at 6am”

In the half-light, Kate is reversing a trailer into the shed. Her mate walks behind the trailer to grab a rake.

What should happen: lights on. Kate uses a spotter she can see, agrees signals first and stops if she loses sight of them. Nobody walks behind a reversing vehicle.

Scenario 3 – “It’s only a few hundred metres”

A mower is needed on the other side of a public road. Driving it there would be quicker than loading the trailer.

What should happen: it doesn’t go on the road unless the Course Superintendent has confirmed the vehicle is registered or permitted and the driver is licensed. Otherwise the supervisor arranges how it is moved.

Scenario 4 – “Dew on the bank”

Early morning, heavy dew. Ali drives the Gator down a grassy bank to check a pump near the dam.

What should happen: drive to the conditions. Park on flat, safe ground well back from the edge and walk the last bit (OHS-17, OHS-18).

# 17. The law in brief

| Law or guidance | What it means for us |
| --- | --- |
| Occupational Health and Safety Act 2004 (Vic) | Employers must, so far as is reasonably practicable, provide safe plant and systems of work, and the information, instruction, training and supervision workers need. Workers must take reasonable care of their own and others’ safety, and cooperate with safety rules. Serious incidents must be reported to WorkSafe straight away, and the scene preserved. |
| Victorian road laws (Department of Transport and Planning) | On public roads, registration, licence and road rules apply. Golf carts and ride-on mowers are only exempt in narrow conditions and must not use footpaths, bike paths or shared paths. Utility vehicles need registration (some can be conditionally registered, with strict conditions) to use a road. |
| WorkSafe Victoria guidance – side-by-side vehicles and ride-on mowers | Fit and keep ROPS up, always wear seatbelts, follow the manufacturer on passengers, don’t let untrained people drive, and don’t bypass safety features. |

# Document control

- Document: OHS-28 Vehicles, Carts & Traffic on Site – Standard Operating Procedure

- Version: 1 – October 2026

- Owner: Course Superintendent

- Review: every 12 months, after any vehicle incident, or when the law changes

- Related documents: OHS-03 Medical Emergency & Ambulance Access · OHS-10 Fuel Handling · OHS-13 Hazard, Incident & Near-Miss Reporting · OHS-14 Pre-start Checks & Out-of-Service Tagging · OHS-16 Working Around Golfers · OHS-17 Slopes & Rollover · OHS-18 Working Near Water · OHS-31 Fit for Work · CHM-01 Pesticide Mixing, Loading & Application · machinery SWPs (mowers, utility vehicles, track loader, excavator, sprayer)

- Based on: WorkSafe Victoria safety alerts on side-by-side vehicles and ride-on mowers, and incident notification guidance; Department of Transport and Planning (Victoria) information on golf cars, ride-on mowers and special work vehicles.