| Issue date: October 2026 | Next review date: October 2027 | Approved by: Andrew Anderson |
| --- | --- | --- |
| Raise the alarm FIRST. Only tackle a fire if it is small and safe to approach, the alarm has been raised, you have a clear way out behind you, and you know how to use the extinguisher. If in doubt, get out and call 000. |  |  |
| Applies to | Everyone who works at Ranfurlie and Amstel – and especially chief wardens, deputies and wardens |  |
| What’s covered | Classes of fire, extinguisher types and colours, fire blankets, electrical, fuel and machinery fires, when to fight and when to get out, the PASS technique, checks and servicing, and what fire wardens do |  |
| Time needed | About 10 minutes to read, then a 9-question knowledge check |  |
| Re-sign | Every 12 months, and whenever this document is updated |  |

# Site details – fill in before issue

These details are specific to each site. Fill in every yellow cell before this procedure is issued, and update them whenever anything changes. The rest of this procedure refers to them.

| Site detail | Ranfurlie | Amstel |
| --- | --- | --- |
| Extinguishers – maintenance shed Location and type of each, e.g. “ABE powder, beside the roller door” |  |  |
| Extinguishers – workshop Location and type of each |  |  |
| Extinguishers – chemical store Location and type (often just outside the door) |  |  |
| Extinguishers – fuel store Location and type |  |  |
| Machinery and vehicles that carry an extinguisher Which machines or vehicles, and the type carried |  |  |
| Fire blankets Locations, e.g. lunchroom kitchen, workshop |  |  |
| Chief warden and deputy chief warden Names and radio call signs |  |  |
| Fire wardens Names and the area each one covers (e.g. shed, workshop, clubhouse) |  |  |
| Warden identification What each warden wears in an emergency, e.g. hat or vest colour |  |  |
| Extinguisher service contractor and last service date Company, phone number and date of the last six-monthly service |  |  |

Key points - Raise the alarm before you fight a fire. Shout “Fire!”, radio it and make sure 000 is called. - Only tackle a small fire – if it is safe, the alarm is raised, you have a clear way out behind you and you know how to use the extinguisher. If in doubt, get out. - Match the extinguisher to the fire. Never use water on fuel, oil, electrical or cooking-oil fires. - PASS: Pull the pin, Aim at the base, Squeeze the handle, Sweep side to side. - Report every extinguisher or fire blanket you use, even a short burst, so it is replaced. - Look at extinguishers as you pass – in place, pin and seal intact, gauge in the green, tag current. - Know your wardens and follow their instructions in an emergency.

# 1. Classes of fire

In Australia, fires are grouped by what is burning. Every extinguisher is labelled with the classes it can be used on.

| Class | What is burning | Examples around the course |
| --- | --- | --- |
| A | Ordinary combustibles – wood, paper, cloth, plastics | Rubbish, pallets, cardboard, dry clippings, seats |
| B | Flammable and combustible liquids | Petrol, diesel, two-stroke, oil, paint, solvents |
| C | Flammable gases | LPG, acetylene and other gas cylinders |
| D | Combustible metals | Magnesium, aluminium shavings – rare; needs special extinguishers and expert advice |
| E | Energised electrical equipment | Switchboards, battery chargers, power tools, the pump station |
| F | Cooking oils and fats | Lunchroom or kitchen cooking |

# 2. Extinguisher types and colours

Australian extinguishers are red, with a coloured band showing the type. Always read the label before you use one.

| Type and colour | Use on | Don’t use on |
| --- | --- | --- |
| Water – all red | A – wood, paper, cloth | Fuel, oil, live electrical, cooking oil – dangerous |
| Foam – blue band | A and B – good on fuel | Live electrical equipment |
| Dry chemical powder (ABE) – white band | A, B, C and E – the most versatile | Cooking oil and fat (F) |
| Dry chemical powder (BE) – white band | B, C, E and F | Wood, paper, cloth (A) |
| Carbon dioxide (CO2) – black band | E – electrical; limited on A and B | Outdoors or in wind, or deep smouldering fires. Can starve you of air in a small closed room. |
| Wet chemical – oatmeal band | F – cooking oil and fat; also A | Live electrical equipment |
| Fire blanket | Small cooking fires; a person whose clothes are on fire | Anything bigger |

ABE and BE powder extinguishers look almost identical – both have a white band. Check the label.

# 3. Fire blankets

- Pull down sharply on the tabs to release the blanket.

- Hold it in front of you by the tabs, with the edges rolled over your hands.

- Place it gently over the fire – don’t throw it – keeping your face and hands behind it.

- Turn off the heat source. Leave the blanket in place for at least 15 minutes, or until the fire brigade arrives.

- Call 000 and ask for Fire – even if the fire is out.

- Report it – a used fire blanket must be replaced.

If someone’s clothes catch fire: stop, drop, cover your face and roll, or smother the flames with a fire blanket. Cool the burn under cool running water for at least 20 minutes, and call 000 for anything but a small burn.

# 4. Fires you are most likely to face here

Electrical fires

- Use CO2 or dry chemical powder. Never water or foam – you could be electrocuted.

- If you can do it safely, switch the power off first.

- Lithium-ion batteries (power tools, battery equipment): if one is smoking, swelling or burning, don’t touch it. Get everyone out, close the door if safe, call 000 and say it is a lithium-ion battery fire. These fires can flare up again hours later.

Fuel and oil fires – workshop and fuel store

- Never use water – it spreads burning fuel.

- Use dry chemical powder or foam. With foam, lay a blanket over the burning liquid rather than blasting into it.

- Stop refuelling and use the fuel store emergency shut-off if safe (OHS-02).

- If the fire is in the fuel store or on a fuel tank, don’t fight it – get everyone well away and call 000.

Mower, Workman and vehicle fires

- Clippings, dust and oil build up around engines, exhausts and bearings and can catch fire. Blow down and clean machines regularly, especially on hot, dry days.

- If a machine catches fire: stop, park brake on, engine off, get off and move well away – upwind if you can. Radio it and call 000.

- Only fight it if it is small and you have the right extinguisher. For an engine fire, don’t lift the hood or engine cover – a rush of air can make it flare. Release the catch only and spray through the gap.

- Smoke from burning plastics, rubber and fuel is toxic – stay out of it.

- A machine fire on dry grass can start a grassfire. On fire danger days, follow OHS-06.

Chemical store fires

Don’t fight it. Burning chemicals give off toxic smoke and containers can burst. Raise the alarm, get everyone out and upwind, call 000 and tell them what is stored there.

# 5. Fight it or get out?

Only tackle a fire if ALL of these are true: - The fire is small and it is safe to approach – not spreading fast, not much smoke. - The alarm has been raised and 000 is being called. - You have a clear way out behind you – the fire is not between you and the exit. - You know how to use the extinguisher, and it is the right type for what is burning. If any answer is no – get out, close the door and call 000. If one extinguisher doesn’t put it out, the fire is too big. Get out.

# 6. Using an extinguisher – PASS

- Check the label is right for this fire, and the gauge (if it has one) is in the green.

- Approach from upwind, walking not running, with your way out behind you.

- P – Pull the pin. Hold the extinguisher steady and pull the pin firmly – don’t squeeze the handle while you pull.

- Give a short test burst away from the fire to check it works and see how far it reaches.

- A – Aim the nozzle at the base of the fire, not the flames.

- S – Squeeze the handle.

- S – Sweep from side to side across the base until the fire is out.

- Back away still facing the fire – it can reignite. Watch for smouldering hot spots.

- Extinguishers empty fast – a 1 kg powder unit lasts less than 10 seconds.

- Hands-on training is the best way to learn. It is good practice, not a legal requirement – CFA recommends it for all staff, not just wardens.

- CO2 has a short range, is loud and very cold, and has no gauge – the test burst is essential.

# 7. After you use one

- Report it to your supervisor and in #ohs – even a short burst or a test. A partly used extinguisher may fail next time.

- Don’t hang a used extinguisher back on its bracket. Give it to your supervisor so it is recharged or replaced.

- Used fire blankets are always replaced.

- If powder went over a machine or electrics, tell the workshop so it can be cleaned and checked.

- Report the fire itself, however small, as an incident (OHS-13).

# 8. Checking extinguishers and fire blankets

Monthly look-over – anyone can do it

Each month a staff member checks every extinguisher and fire blanket in their area. Report anything wrong to your supervisor straight away.

| Check | What to look for |
| --- | --- |
| In place | On its bracket, with the sign above it |
| Easy to reach | Nothing in front of it – no machines, pallets, bags or hoses |
| Pin and seal | Safety pin in, tamper seal unbroken |
| Gauge | Needle in the green (CO2 has no gauge) |
| Condition | No dents, rust, leaks, cracked hose or blocked nozzle; label readable |
| Service tag | Tag attached, showing a service within the last six months |
| Fire blanket | Container intact, both tabs hanging down |

Six-monthly service – by a competent person

Club practice is to have extinguishers and fire blankets inspected and serviced every six months by a competent fire equipment technician, following Australian Standard AS 1851, with more detailed yearly and five-yearly servicing. Each service is recorded on the maintenance tag. The contractor and last service date are in Site details. Never refill, repair or recharge an extinguisher yourself.

# 9. Fire wardens

Each site has an emergency control organisation – a chief warden, a deputy chief warden and wardens – set up under Australian Standard AS 3745, Planning for emergencies in facilities. Their names, areas and what they wear are in Site details.

In an emergency, follow the wardens’ instructions – even if they aren’t normally your boss. Under AS 3745, wardens’ instructions take precedence over the normal management structure during an emergency. That includes managers, contractors and visitors.

| Role | What they do in an emergency |
| --- | --- |
| Chief warden | - Takes charge when the alarm is raised and decides what happens – evacuate, shelter or stay - Makes sure 000 is called and someone is meeting emergency services - Directs the wardens and gets the head count - Briefs emergency services when they arrive and hands over control - Decides, with emergency services, when it is safe to go back in |
| Deputy chief warden | Steps in when the chief warden is away or busy. Helps with the radio and the head count. |
| Wardens | - Check their area is clear – shed, workshop, lunchroom, toilets, chemical store – without putting themselves at risk - Close doors (not lock them) to slow fire and smoke - Direct people to the assembly point and help anyone who needs it - Use an extinguisher on a small fire only if it is safe and they know how to use it - Report to the chief warden: “Workshop clear” – or who is missing |
| Everyone | - Know the alarm, the exits and your assembly point - Know who your wardens are and how to spot them - Follow warden instructions straight away - Report to your warden at the assembly point and don’t leave without telling them |

Day to day, wardens also

- Keep exits, paths and extinguishers clear in their area

- Report used, missing or damaged fire equipment

- Show new starters the exits, extinguishers and assembly point

- Take part in warden training and evacuation drills

If you are asked to be a warden, the Club will train you for the role. When a warden is away, the chief warden names who covers their area.

# 10. Preventing fires

- Keep the shed and workshop tidy – no rags, cardboard or clippings piled near machines, chargers or switchboards.

- Put oily rags in a closed metal bin.

- Keep fuel in proper fuel containers, in the fuel store – never in the lunchroom or office.

- No smoking or naked flames near fuel, chemicals or gas.

- Blow down machines after mowing, especially around exhausts and engines.

- Welding and grinding only with the area cleared, an extinguisher at hand, and a check for smouldering afterwards.

- Charge batteries on a clear, non-flammable surface away from fuel, and unplug them when charged.

# 11. Scenarios – what should happen?

Scenario 1 – “Smoke from the rough mower”

On a hot afternoon, Kai sees smoke and small flames coming from under the engine cover of his rotary mower in the rough.

What should happen: Kai stops, puts the park brake on, turns the engine off, gets off and moves away upwind. He makes the emergency call and 000 is called – it could start a grassfire. If the fire is still small and he has a powder extinguisher, he releases the cover catch without lifting it and sprays through the gap. If it grows, he gets well away. The extinguisher is reported and replaced.

Scenario 2 – “Sparks at the charger”

A battery charger on the workshop bench starts sparking and smoking.

What should happen: no water. If safe, switch it off at the power point or switchboard, and use CO2 or powder on a small fire. If a lithium-ion battery is burning, don’t touch it – everyone out, door closed, 000 told it is a lithium-ion battery fire.

Scenario 3 – “Hung back up”

Tom used part of the workshop extinguisher on a small rag fire last week, put it back on the bracket and didn’t tell anyone.

What should happen: any use is reported so the extinguisher is recharged or replaced, and the fire is reported as an incident. A half-empty extinguisher could fail the next person.

# 12. The law in brief

| Law or standard | What it means for us |
| --- | --- |
| Occupational Health and Safety Act 2004 (Vic) | Employers must, so far as is reasonably practicable, provide a safe workplace, safe systems of work, and the information, instruction and training staff need – including how to respond to a fire. Workers must take reasonable care and cooperate with safety arrangements. |
| Dangerous Goods (Storage and Handling) Regulations 2022 (Vic) | Where flammable or combustible liquids such as fuel are stored above “minor storage” quantities, fire protection must be provided – which may include extinguishers. Where dangerous goods exceed the higher “fire protection quantity”, WorkSafe says fire protection systems are designed with the fire authority and firefighting equipment is maintained to AS 1851-2012. |
| Building law – essential safety measures | Portable extinguishers and fire blankets in buildings are essential safety measures and must be maintained, with records kept. |
| OHS Act – notifiable incidents | A fire that exposes someone to serious risk to their health or safety must be notified to WorkSafe immediately (13 23 60). See OHS-13. |
| Australian Standards | AS 2444 – choosing and locating extinguishers and fire blankets. AS/NZS 1841 – extinguisher types and colours. AS 1851 – routine servicing. AS 3745 – emergency planning, including wardens. |

# Document control

- Document: OHS-07 Fire Extinguishers & Fire Wardens – Standard Operating Procedure

- Version: 1 – October 2026

- Owner: Course Superintendent

- Review: every 12 months, after any fire or emergency, or when the law changes

- Related documents: OHS-01 · OHS-02 · OHS-06 · OHS-10 · OHS-13 · OHS-14 · OHS-23 · CHM-03

- Based on: CFA guidance on fire extinguishers, fire blankets and machinery fires; Fire Rescue Victoria vehicle fire advice; Queensland Fire and Emergency Services and Fire and Rescue NSW extinguisher guidance; AS 2444, AS/NZS 1841, AS 1851-2012 and AS 3745-2010; WorkSafe Victoria guidance on dangerous goods and notifiable incidents; ANZCOR burns guideline.